DEFINE · STAGE 02 · GATEWAY 2 COMMITMENT

Engineering Product Requirements Document

Twin-Tank Atmospheric Water Heater System

| Document | Engineering Product Requirements Document (PRD) |
| --- | --- |
| Product | Twin-Tank Atmospheric Water Heater System |
| Stage | Define (Stage 02) — Gateway 2 commitment document |
| Revision | Rev A — Sample / draft for review |
| Upstream inputs | VPC, BMC, QFD Level 1, DFMEA Steps 1–3 with DSM, Future-state journey, Brainstorm with unmet-needs lenses |
| Downstream consumers | Develop stage — component decomposition, QFD L2/L3, BOM, DFMEA 4–7, CAD, prototypes |
| Convention | MoSCoW prioritization: Must (release blocker), Should (release goal), Could (stretch). Every requirement has a unique ID, a verifiable acceptance criterion, and a traceability link upstream and downstream. |
| Approval gate | This PRD, together with the DFMEA 1–3 + DSM document and the Final ID render, constitutes the Gateway 2 commitment bundle. Sign-off freezes the requirements baseline for Develop entry. |

# 1.  Executive summary

This PRD specifies the requirements for a domestic appliance that captures water from ambient humid air, heats and stores it across two tanks, and delivers hot water on demand. It is the Gateway 2 commitment document for the Define stage: together with the DFMEA Steps 1–3 (with DSM coupling analysis) and the final industrial-design render, it constitutes the requirements baseline that the Develop stage commits against.

Three things make this product non-trivial. First, the architecture is a coupled core: three subsystems — Tank & Pressure Management, Water Flow & Delivery, and Sensing & Control — depend bidirectionally on each other and must be co-developed. Second, safety is achieved through redundant independent paths, not through a single more-reliable path. Third, the value proposition combines source resilience (atmospheric capture + mains backup) with energy efficiency (twin-tank thermal management) — neither incumbent category does both.

The quality and engineering requirements in §8 are derived from QFD Level 1 and carry numeric targets; several target values remain TBD and are tracked in §14. Feature-level requirements with concrete measurable targets are catalogued in §7. The functional requirements in §6 trace to the F-01 through F-14 functions identified in DFMEA Step 3. Action Priority H items from DFMEA must be closed or formally deferred before any Proto 2 design freeze.

# 2.  Product overview

| The problem | Households in regions with intermittent municipal water and high humidity face two coupled problems: (1) unreliable hot-water availability when mains supply is interrupted, and (2) high energy bills from conventional electric storage water heaters that lose heat to ambient and over-cycle on standby. Existing atmospheric water generators address potable water but not hot-water-on-demand; existing smart water heaters address energy but not source resilience. |
| --- | --- |
| The product | A domestic appliance that captures water from ambient humid air, heats and stores it across two tanks, and delivers hot water on demand. Onboard sensing manages temperature, pressure, and level; a mobile app surfaces status and alerts; the enclosure is service-friendly with documented access for descaling, drain, and component replacement. |
| Why now | Condensation-based water capture has matured cost-wise; embedded ML for usage-pattern learning is feasible on commodity MCUs; consumer acceptance of connected appliances is well past the early-adopter phase in target markets. |
| Why us | The competitive frame in adjacent categories (smart thermostats, instant water heaters, AWGs) is feature-led, not architecture-led. A twin-tank thermal-management architecture lets us deliver both source resilience and energy efficiency in one envelope — neither incumbent category does both. |
| Primary value proposition | Hot water that is always available, costs less to run, and tells you when it needs service before it fails. |

# 3.  Goals and non-goals

## 3.1  Goals

- Deliver hot water on demand to a single-household installation with availability targets that exceed conventional electric storage water heaters under intermittent-mains conditions.

- Cut annual energy consumption per litre of delivered hot water versus the incumbent electric storage baseline in the same usage profile.

- Provide a service experience where routine maintenance can be completed by the homeowner without specialist tools, and qualified service requires no more than one site visit per repair.

- Surface predictive maintenance alerts before failures cause loss of service.

- Pass all in-scope regulatory and safety standards on first formal certification submission.

## 3.2  Non-goals (this release)

- Outdoor-rated unsheltered installation (Proto 1/2 assumes a sheltered domestic enclosure).

- Multi-household / multi-tenant logic or shared-app accounts.

- Cloud connectivity, OTA firmware updates, voice-assistant integration (planned post-launch).

- Industrial or commercial-scale capacity beyond a single household's daily envelope.

- Off-grid solar-only operation in baseline product; solar input is a future variant.

# 4.  Personas and jobs to be done

Personas are reduced to those whose needs directly produce requirements in §6–§9. Sub-segmentation (e.g., household size, regional usage profile) is handled in the QFD weighting rather than by adding more personas.

| Persona | Primary job-to-be-done | What success looks like for them |
| --- | --- | --- |
| Primary user — head of household | "When I open the tap, give me hot water without me thinking about it." | Hot water available on every reasonable use occasion; bill goes down vs. the old heater; one app notification per quarter or fewer. |
| Secondary user — household members | "Use hot water without learning a new appliance." | Existing taps behave the same as before; no app required for daily use; local display readable at a glance. |
| Installer | "Install it in one visit, no surprises, sign off." | Single-visit install with standard tools; clear mounting templates; pre-commissioning self-test pass before leaving site. |
| Service technician | "Diagnose, replace the right part, leave it working." | Diagnostic codes lead to one root cause; serviceable parts are accessible without dismantling unrelated subsystems; service log is retrievable on-site via QR code. |

# 5.  User experience requirements

UX requirements describe how the user perceives and interacts with the system. They constrain the UI & Connectivity subsystem and parts of Sensing & Control. Each UX-xx item has an acceptance criterion that can be checked by inspection or user trial.

| ID | Requirement | Priority | Owner subsystem(s) |
| --- | --- | --- | --- |
| UX-01 | Local display readable at arm's length in normal household lighting without requiring the user to bend down. | Must | Sensing & Control + UI & Connectivity |
| UX-02 | First-run setup completable in under 10 minutes including app pairing, location, and basic schedule. | Must | UI & Connectivity |
| UX-03 | Alerts are categorized by user action required: informational / action-needed / urgent. Urgent alerts trigger both local and app notification. | Must | Sensing & Control + UI & Connectivity |
| UX-04 | The mobile app exposes status, daily/weekly history, and service log, and is not required for daily appliance use. | Should | UI & Connectivity |
| UX-05 | User can mute non-urgent alerts for a chosen window (e.g., overnight) without disabling urgent alerts. | Should | UI & Connectivity |
| UX-06 | Accessibility — display supports large-text mode and the app meets WCAG 2.1 AA for the principal user flows. | Should | UI & Connectivity |

# 6.  Functional requirements

Functional requirements describe what the system does. Each FR-xx realizes one or more numbered functions (F-01 through F-14) identified in DFMEA Step 3 Function Analysis, and each is owned by one or more subsystems. The owner-subsystem column maps to the abbreviations used in DFMEA §2.3 (TPM, WFD, SC, SM, EI, UIC).

| ID | Realizes function | Requirement | Priority | Owner subsystem(s) |
| --- | --- | --- | --- | --- |
| FR-01 | F-01 | The system shall store water across two tanks with isolation between them so that one tank can be drained or serviced while the other continues to deliver hot water. | Must | TPM |
| FR-02 | F-01 | The system shall maintain stored water within a configurable target temperature band, with default values appropriate for domestic hot-water use. | Must | TPM + SC |
| FR-03 | F-02 | The system shall protect against overpressure by means independent of the main control MCU. | Must | TPM |
| FR-04 | F-02 | The system shall damp hydraulic surges from pump cycling and demand spikes such that no audible water hammer is detectable at the user-side outlet under specified test conditions. | Should | TPM + WFD |
| FR-05 | F-04 | The system shall capture moisture from inlet ambient air via the condensation module, with capture rate quantified per the QFD targets in §8. | Must | WFD |
| FR-06 | F-04 | The system shall accept mains / make-up water as a secondary source when atmospheric capture is insufficient to maintain a configurable minimum reserve. | Must | WFD |
| FR-07 | F-04 | The system shall deliver hot water at the outlet within a specified flow and pressure envelope under nominal operating conditions. | Must | WFD + TPM |
| FR-08 | F-05 to F-09 | The system shall measure tank temperature, tank pressure, tank level, and ambient humidity with accuracy and refresh rate sufficient to support the safety interlocks in FR-11 through FR-13. | Must | SC |
| FR-09 | F-11 | The system shall close control loops for heating, pumping, and valve actuation in a deterministic manner that meets the latency budget in §8. | Must | SC |
| FR-10 | F-11 | The system shall provide diagnostic data, error codes, and event logs accessible both locally and via the mobile app. | Must | SC + UIC |
| FR-11 | F-11 | The system shall trip an over-temperature cutoff if measured tank temperature exceeds the configured safety limit, by a path that does not depend on a functioning main MCU firmware loop. | Must | SC + TPM |
| FR-12 | F-11 | The system shall prevent the pump from running dry by interlock; dry-run condition must remove pump power within a bounded time of detection. | Must | SC + WFD |
| FR-13 | F-11 | The system shall trigger overpressure relief through an independent path (mechanical relief + independent electrical cutoff) when measured pressure exceeds the configured safety threshold. | Must | TPM + SC |
| FR-14 | F-03 | The system shall provide service access to tanks, pumps, valves, sensors, and electronics such that each can be inspected and replaced without dismantling unrelated subsystems. | Must | SM + EI |
| FR-15 | F-13 | The system shall surface service intervals and predictive maintenance prompts based on measured wear indicators (cycle counts, drift, calibration deviation). | Should | SC + UIC + SM |
| FR-16 | F-10 | The system shall provide a local display showing at minimum: power state, current mode, tank levels, last alert, and a service-status indicator. | Must | UIC |
| FR-17 | F-14 | The system shall pair with a single mobile app instance via Bluetooth; subsequent re-pairings of additional devices require explicit confirmation from the existing primary device. | Should | UIC |
| FR-18 | F-12 | The enclosure shall retain thermal energy in the stored hot water such that standby losses meet the QFD targets in §8. | Must | EI |

# 7.  Feature-level requirements

This section catalogues the named, concrete features that the product is being built to deliver, each with a measurable target. It sits between the abstract functional requirements in §6 and the QFD-derived engineering targets in §8: a feature describes a specific design solution ("non-clogging open vent", "PIR-based wake-up", "BEE 5-star insulation buffer") chosen to realize one or more functions, with a numeric acceptance threshold.

ID convention: features are numbered FT-01 through FT-15 to avoid collision with the F-01 through F-14 subsystem-function IDs used in DFMEA Step 3 and referenced in §6. A feature may realize one or more F-xx functions and is owned by one or more subsystems.

| ID | Feature name | Detailed feature requirement | Measurable target | Owner subsystem(s) | Pri |
| --- | --- | --- | --- | --- | --- |
| FT-01 | Atmospheric Venting Integrity | Non-clogging, open-to-atmosphere vent assembly that maintains zero gauge pressure in both tanks during heating cycles, eliminating catastrophic burst risks. | Max internal tank pressure: 0 kPa (0 bar) | TPM | Must |
| FT-02 | Twin-Tank Thermal Stratification | Optimize fluid transfer and multi-tank flow dynamics so that cold inlet water filling Tank 1 does not rapidly degrade or mix with ready-to-use hot water in Tank 2. | First-hour hot-water delivery efficiency: > 90% | TPM + WFD | Must |
| FT-03 | Service-First Modular Access | Top-mounted or front-facing quick-release service hatches allow field personnel to clear sediment or perform high-flow chemical flush without dismantling core plumbing. | Mean Time To Service (MTTS) for descaling: < 20 min | SM + EI | Must |
| FT-04 | Booster Flow Integration | Because the open-atmosphere design operates at zero line pressure, the outlet architecture interfaces with internal micro-boosters or external home pumps to ensure high-pressure delivery at the tap. | Minimum discharge flow rate at tap: 12 L/min | WFD | Must |
| FT-05 | Dual-Tank Level Electronic Sensing | Replace legacy pressure-switch safety logic with continuous electronic level monitoring (robust float arrays or non-contact ultrasonic sensors) in both tanks to prevent dry-heating and handle multi-stage filling. | Dry-run protection response latency: < 500 ms | SC + WFD | Must |
| FT-06 | "Always Available" Agentic Logic | Heating logic prioritizes maintaining a minimum volume of ready hot water in Tank 2, dynamically adjusting heating cycles based on learned family usage patterns. | "Zero cold shower" success rate: > 95% | SC + UIC | Must |
| FT-07 | PIR-Sensor Based Wake-Up | Passive infrared sensors in the bathroom environment detect occupancy, allowing the system to wake or throttle heating cycles proactively to eliminate idle standing losses. | Idle standing energy losses avoided: 15% reduction | SC | Should |
| FT-08 | Multi-Node Thermal Safety Array | Distributed NTC thermistor array tracks fluid temperatures across both tanks to prevent localized boiling and minimize temperature spikes where scale precipitates. | Temperature sensor calibration accuracy: ± 0.5 °C | SC + TPM | Must |
| FT-09 | Vulnerable Mode Maximum Temp Cap | Software-defined cap or automated mixing loop restricts outlet water temperature to a maximum of 49 °C when vulnerable mode (elderly / children) is activated by the user. | Vulnerable mode max temp: 49 °C | SC + UIC | Must |
| FT-10 | Independent Physical Overrides | Independent, non-software-dependent physical thermal safety switches (bimetallic cutouts) on heating elements provide redundant over-temperature control. | Thermal safety cutout activation: 75 °C | TPM + SC | Must |
| FT-11 | Bluetooth-First Offline Core Logic | Heating schedules, level sensing, and safety sequences are managed locally on the device (ESP32-C6 BT) to ensure core functionality during Wi-Fi or cloud outages. | Offline logic response latency: 0 ms (local) | SC + UIC | Must |
| FT-12 | BEE 5-Star Standing Loss Buffer | Certified BEE 5-star validation achieved by keeping standing loss to a maximum of 0.342 kWh/24h via high-density PUF insulation tailored around the twin-tank geometry. | BEE-certified standing loss: ≤ 0.342 kWh/24h | EI + TPM | Must |
| FT-13 | On-Tank QR Code AR Service Guides | Weather-resistant QR codes on the outer chassis launch AR-guided exploded views of the twin-tank layout and step-by-step local descaling procedures for service technicians. | AR guide access success rate: > 98% | SM + UIC + EI | Should |
| FT-14 | One-Click UC API Maintenance Hook | Companion app integrates with Urban Company API. When the localized sensor array detects high sediment accumulation in the modular scale trap, a push notification offers a one-click pre-booked descaling appointment. | Automated booking integration latency: < 3 s | UIC + SC + SM | Should |
| FT-15 | Uniform 20-Min Toolless Service Layout | Internal fluid plumbing, modular scale trap, and open-atmosphere dual tank lids use standardized toolless thumb-screws and universal seals optimized for rapid gig-professional turnaround. | Maximum descaling maintenance window: 20 min | SM + EI | Must |

# 8.  Quality and engineering requirements (from QFD Level 1)

This section contains the engineering characteristics — the QFD "Hows" — that translate customer needs into measurable targets. Each QR-xx requirement has a target value, a test or tolerance basis, and a traceable origin in the customer-need column. Where a QR-xx target overlaps with a feature target in §7 (for example, dry-run latency, standing loss, vulnerable-mode temperature cap), the §7 value is authoritative and the QR-xx row references it for traceability.

Where target values are written as "target X" with no number, the numeric value is TBD pending QFD Level 1 refinement and is tracked in §14.

| ID | Engineering characteristic (QFD How) | Target value | Test / tolerance basis | Priority | Customer need (QFD What) |
| --- | --- | --- | --- | --- | --- |
| QR-01 | Hot-water availability (uptime under intermittent mains) | ≥ 99% of demand events met | Measured over 30-day field cycle | Must | "Always available" need |
| QR-02 | Atmospheric water capture rate at design ambient (28°C, 70% RH) | ≥ design target litres/day (TBD per QFD L1) | Per-day rolling average; minimum daily floor specified separately | Must | Source resilience need |
| QR-03 | Energy efficiency — kWh per litre delivered at 55°C | ≤ target value kWh/L (TBD per QFD L1) | Standby + active envelope, 24h test profile | Must | "Costs less to run" need |
| QR-04 | Standby thermal loss | ≤ target W at 55°C tank, 20°C ambient | Per insulation spec; verified by 24h passive cooldown test | Must | Efficiency need |
| QR-05 | Time to first hot water from cold start | ≤ target minutes to first 55°C delivery | Cold start = both tanks at ambient | Should | Convenience need |
| QR-06 | Delivery pressure at outlet under nominal demand | Within ±10% of mains-equivalent baseline | Measured at outlet with rated flow | Must | "No surprises at the tap" need |
| QR-07 | Delivery flow rate at outlet under nominal demand | ≥ target L/min at specified outlet pressure | Measured at standard test condition | Must | "Behaves like normal plumbing" need |
| QR-08 | Pressure relief activation accuracy | Cracking pressure within ±5% of nominal setpoint | Per pressure-vessel safety standard | Must | Safety (regulatory) |
| QR-09 | Over-temperature cutoff response time | ≤ target seconds from threshold crossing to heater de-energized | Independent of MCU firmware loop | Must | Safety (regulatory) |
| QR-10 | Dry-run protection response time | ≤ target seconds from low-level event to pump de-energized | Verified by feed-cut trial | Must | Safety + reliability |
| QR-11 | Acoustic emission at user-side outlet | ≤ target dB(A) under rated flow | 1 m from outlet, A-weighted | Should | "Quiet enough" need |
| QR-12 | Conducted and radiated EMI | Compliant with IEC 61000-6-2 (immunity) and 6-3 (emissions) for residential equipment | Per applicable regional standards | Must | Regulatory |
| QR-13 | Ingress protection rating | IPX4 minimum for the assembled enclosure | Sheltered installation; condensation-aware | Must | Reliability under household conditions |
| QR-14 | Mean time between service interventions | ≥ target months under nominal usage profile | Defined per service log and field-failure data | Should | Service cost-of-ownership |
| QR-15 | Time to complete homeowner-serviceable maintenance (descaling, filter) | ≤ target minutes per intervention | Without specialist tools | Should | Serviceability |
| QR-16 | Time to complete one technician-serviceable repair (replace pump or sensor) | ≤ target minutes per repair | With standard tools, single-visit | Should | Serviceability |
| QR-17 | Bluetooth pairing reliability | ≥ 95% successful first-pair attempts under specified RF environment | Standard household RF noise floor | Should | Install experience |
| QR-18 | Sensor accuracy — tank temperature | ± target °C across operating range | After warm-up and self-calibration | Must | Closed-loop control + safety |
| QR-19 | Sensor accuracy — tank pressure | ± target % FS across operating range | Required for overpressure trip path | Must | Closed-loop control + safety |
| QR-20 | Control-loop end-to-end latency (sensor read → actuator update) | ≤ target ms under worst-case load | Determinism budget for FR-09 | Must | Closed-loop control |

# 9.  Non-functional requirements

Non-functional requirements cover safety, reliability, EMC, environmental tolerance, privacy, maintainability, and internationalization. Safety items NFR-01 through NFR-04 are release blockers because they tie to regulatory certification. NFR-01 ties directly to the AIAG-VDA Action Priority discipline applied in DFMEA Steps 4–7.

| ID | Category | Requirement | Priority |
| --- | --- | --- | --- |
| NFR-01 | Safety | All identified Action Priority H items from DFMEA must be either closed or formally deferred with documented rationale prior to any Proto 2 design freeze. | Must |
| NFR-02 | Safety | The system shall comply with the applicable regional pressure-vessel safety standard for stored hot water. | Must |
| NFR-03 | Safety | The system shall comply with the applicable regional electrical safety standard for domestic appliances. | Must |
| NFR-04 | Safety | The system shall comply with the applicable potable-water-contact material standards for all wetted parts. | Must |
| NFR-05 | Reliability | The mean time between safety-relevant failures (any H-rated failure mode) shall meet or exceed the target stated in QR section. | Must |
| NFR-06 | Reliability | All H-rated and M-rated DFMEA items shall have a defined design validation parameter and a test method specified in the Integrated Tracker. | Must |
| NFR-07 | EMC | The system shall meet IEC 61000-6-2 (immunity) and 6-3 (emissions) for residential equipment, with engineering margin documented. | Must |
| NFR-08 | Environmental | The system shall operate within specified ambient temperature and humidity ranges for sheltered domestic installations. | Must |
| NFR-09 | Environmental | The system shall manage internal condensation such that no liquid water accumulates in electronics or sensor cavities under any defined operating condition. | Must |
| NFR-10 | Privacy & data | User data captured by the app (usage patterns, service logs) shall remain on-device by default; any future cloud upload is opt-in. | Must |
| NFR-11 | Privacy & data | Bluetooth pairing shall use a current-best-practice secure pairing scheme; unpaired devices cannot read system data. | Must |
| NFR-12 | Maintainability | Firmware shall log faults to non-volatile memory in a structured, retrievable format; logs survive power cycles. | Must |
| NFR-13 | Maintainability | Service interfaces (diagnostic port or app-side equivalent) shall expose error codes mapped to a published code book. | Should |
| NFR-14 | Internationalization | The local display and app shall support display in the languages required for the launch markets. | Should |

# 10.  Architecture overview

This section is non-normative — it describes the architectural posture that the requirements above assume. The authoritative architecture documents are the DFMEA Steps 1–3 (with DSM) artifact and, in due course, the component-decomposition output of the Develop stage.

| Architectural posture | Modular at the subsystem boundary; tightly integrated within the coupled core. Six subsystems own non-overlapping responsibilities (per DFMEA §2.2), but the three core subsystems must be co-developed against shared interface control documents. |
| --- | --- |
| Coupled core (DSM finding) | Tank & Pressure Management, Water Flow & Delivery, and Sensing & Control form a bidirectionally coupled cluster. Develop-stage work begins by decomposing all three in parallel, with ICDs locked before component selection. See DFMEA §2.3 for the full coupling map and §2.3.4 for ICD priorities. |
| Safety architecture | Two independent safety paths: (a) mechanical pressure relief, independent of any electrical path; (b) over-temperature cutoff that does not depend on a functioning main MCU firmware loop. These are required by FR-11 and FR-13 and constrain component selection in Develop. |
| Control architecture | A single primary MCU runs the closed loops; safety interlocks run on independent paths so MCU failure does not defeat safety. The control loop latency budget (QR-20) is allocated across sensor refresh, MCU processing, and actuator response. |
| Service architecture | Service & Maintenance is treated as a first-class subsystem rather than an afterthought. Access panels, isolation valves, and modular replaceability are constraints on the enclosure and routing, not consequences. This drives FR-14 through FR-17 and the QR-14 through QR-16 quality targets. |

# 11.  Constraints and assumptions

Constraints are externally imposed limits the design must respect. Assumptions are statements taken as true at the time of writing this PRD; if any assumption is invalidated, the requirements that depend on it must be re-examined.

| ID | Type | Statement |
| --- | --- | --- |
| C-01 | Constraint | Bluetooth-only connectivity for Proto 1/2. Cloud and OTA are out of scope for the baseline product. |
| C-02 | Constraint | Single-household installation. No multi-tenant logic, no shared accounts. |
| C-03 | Constraint | Sheltered domestic installation; outdoor unsheltered deployment is a future variant, not a baseline requirement. |
| C-04 | Constraint | Component-level decisions deferred to Develop; the PRD does not specify part numbers. |
| A-01 | Assumption | Mains water is available as make-up supply for the target launch markets. |
| A-02 | Assumption | Ambient humidity in the target launch markets is sufficient to make atmospheric capture meaningful as a primary or supplementary source. |
| A-03 | Assumption | Households in the target markets have a smartphone in the household capable of running the mobile app. |
| A-04 | Assumption | Field-failure data from conventional electric storage water heaters and AWGs is a valid input to DFMEA Step 1 lessons-learned. |

# 12.  Success metrics and validation strategy

Each metric traces back to one or more requirement IDs, has a defined validation method, and a success threshold that distinguishes pass from fail. The Develop stage is responsible for instantiating these into specific test plans; the PRD only fixes the metric and the threshold.

| Metric | Traces to | Validation method | Success threshold |
| --- | --- | --- | --- |
| Hot-water availability | QR-01 | Field cycle over 30 days in target market homes | ≥ 99% demand events met |
| Energy per delivered litre | QR-03 | 24h standardized usage profile, instrumented | Improvement vs. baseline electric storage heater |
| First-pair success rate | QR-17 | Out-of-box install trials with naive users | ≥ 95% on first attempt |
| Safety AP closure | NFR-01, NFR-06 | DFMEA Integrated Tracker review | 100% of H-rated items closed or formally deferred |
| Single-visit repair rate | QR-16 | Service-call audit during field trial | ≥ 80% of repairs completed in one visit |
| Predictive maintenance hit rate | FR-15 | Track alerts vs. actual failures over field trial | ≥ 70% of failures preceded by an alert |

# 13.  Out of scope and future work

- Cloud connectivity, OTA firmware updates, voice-assistant integration — planned post-launch.

- Solar-input variant for off-grid or grid-supplement installations — future variant.

- Outdoor unsheltered deployment — future variant with weatherproofing scope.

- Multi-household / multi-tenant logic — not planned for current product family.

- Software FMEA — to be scoped as a separate exercise once firmware architecture stabilizes.

- Industrial / commercial-scale capacity — separate product line, not covered here.

# 14.  Open questions and TBDs

These items are open at the time of Gateway 2 review. Each must be closed in the first Develop review cycle or formally deferred with documented rationale.

- Final numeric targets for all QR-xx target values (TBD — pending QFD L1 refinement pass).

- Sensor accuracy budgets (QR-18, QR-19) — to be allocated against control-loop latency budget (QR-20) and overpressure trip response (QR-09).

- Hot-water capacity profile per household tier — final segmentation choice affects tank-volume targets in QR-02.

- Acoustic limit (QR-11) — pending competitive benchmarking against incumbent units in-market.

- Service interval target (QR-14) — pending input from initial Proto 1 wear data.

- Confirmation that the regional EMC standards listed in NFR-07 cover all target launch markets without per-market amendment.

- Decision on whether NFR-14 (internationalization) is Should or Must for first launch — depends on launch-market selection.

# 15.  Glossary

| Term | Definition |
| --- | --- |
| AP | Action Priority (AIAG-VDA 2019). Replaces legacy RPN. H/M/L lookup based on Severity, Occurrence, Detection. |
| AIAG-VDA | Joint FMEA Handbook published by AIAG and VDA in 2019; current best-practice methodology for DFMEA. |
| BMC | Business Model Canvas. |
| DFMEA | Design Failure Mode and Effects Analysis. |
| DSM | Design Structure Matrix. A square matrix that exposes coupling between elements of a design. |
| F-01 through F-14 | Numbered functions identified in DFMEA Step 3 Function Analysis. Each PRD functional requirement realizes one or more F-xx. |
| ICD | Interface Control Document. Specifies the exchanges across an interface between two subsystems. |
| MoSCoW | Must / Should / Could / Won't prioritization. This PRD uses Must / Should / Could; Won't items appear in §4 non-goals. |
| PRD | Product Requirements Document. |
| QFD | Quality Function Deployment. The method used to translate customer needs (Whats) into engineering characteristics (Hows) with priority weighting. |
| RPN | Risk Priority Number = Severity × Occurrence × Detection. Legacy FMEA metric; superseded by AP in AIAG-VDA 2019. |
| VPC | Value Proposition Canvas. |

— End of document —